CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Alex Cater

Student #: 2000583031

Student Email: cater@unlv.nevada.edu

Primary Github address: https://github.com/katmando

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB Xplained Mini

Micro USB Cable (Power Supply)

Block Diagram N/A (Everything done through Atmega328PB Xplained Mini)

Figure 1 – Flow Chart for Coding Algorithm

- INITIAL/DEVELOPED CODE OF TASK

#define F_CPU 16000000UL	// Frequency of Xplained Mini (16MHz)

#include <avr/io.h>		// Standard AVR Library

#include <stdio.h>		// AVR library containing printf functions

#include <avr/interrupt.h>	// AVR library containing interrupt functions

#include <util/delay.h>	// AVR library containing _delay_ms() function

#define BAUDRATE 9600			// Baudrate in Bits per second (bps)

#define BAUD_PRESCALLER ((F_CPU / (BAUDRATE * 16UL)) - 1)	// Baudrate Prescaler UBRR0

#define MAX_1s 15624				// 1s delay: OCR1A = (16MHz*1000ms/prescaler) -1

//Declaration of our functions

void Timer1_init(void);			// Function to initialize Timer1

void USART_init(void);			// Function to initialize USART

unsigned char USART_receive(void);		// Function to receive Serial data from UDR0

void USART_send( unsigned char data);	// Function to send individual char into UDR0

void USART_putstring(char* StringPtr);	// Function to break string into chars and send

char String[] = "DA3A";	// 'String[]' is an array treated as a string when between " "

char outs[20];		// 'outs[]' to store integer/float values into array of chars

char t[10];			// 't[]' use

unsigned int rand_int;	// contains integer representing random value

float rand_float;		// contains float representing arbitrary random value

int main(void) {

TIMSK1 |= (1 << OCIE1A);		// Set Interrupt on Compare Match

sei();					// Enable Global Interrupts

Timer1_init();			// Call the Timer1 initialization code

USART_init();				// Call the USART initialization code

while (1) {				// Infinite loop

if (TCNT1 == OCR1A) {	// Display a String when Timer1 Matches OCR1A

rand_int = rand();	// Randomize Integer Value

rand_float = 4.716;	// Arbitrary Float Value chosen

USART_putstring(String);	// Pass 'String' to send serial of chars

USART_send(' ');		// Puts blank space into terminal monitor

snprintf(outs,sizeof(outs),"%3d", rand_int);	// Stores integer

// 'rand_int' into the string 'outs'

USART_putstring(outs);	// Pass 'outs' to to send serial of chars

USART_send(' ');		// Puts blank space into terminal monitor

dtostrf(rand_float, 3, 5, t);	// Store float readable to

// 'snprintf'

snprintf(outs,sizeof(outs),"%s", t);		// Stores 't' (float

// 'rand_float') into the string 'outs'

USART_putstring(outs);	// Pass 'outs' to send serial of chars

USART_send('\n');		// Puts new line into terminal monitor

}

}

return 0;

}

void Timer1_init(void) {		// Function to Initialize Timer1 properties

OCR1A = MAX_1s;		// Let the OCR1A be the Max Value used for 1s delay

TCCR1A = (0<<COM1A1)|(0<<COM1A0);	// Normal Operation, Disconnected OC1A COM1A1:0

TCCR1A = (0<<COM1B1)|(0<<COM1B0);	// Normal Operation, Disconnected OC1B COM1B1:0

TCCR1A = (0<<WGM11)|(0<<WGM10);		// Set Timer1 to CTC Mode 'WGM1:0'

TCCR1B = (0<<WGM13)|(1<<WGM12);		// Set Timer1 to CTC Mode 'WGM3:2'

TCCR1B = (1<<CS12)|(0<<CS11)|(1<<CS10);	// Set Clock Select for Prescaler '1024'

}

ISR(TIMER1_COMPA_vect) {	// Function to Truncate Timer1 to delay properly for 1 second

TCNT1 = 0;		// Restart Timer1 from the Beginning

return;		// Resume code from where interrupt left off

}

void USART_init(void) {			// Function to Initialize USART properties

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);	// Store Upper Baudrate into UBRR0H

UBRR0L = (uint8_t)(BAUD_PRESCALLER);	// Store Lower Baudrate into UBRR0L

UCSR0B = (1 << RXEN0) | (1 << TXEN0);	// Enable Receiver and Enable Transmitter

UCSR0C = (3 << UCSZ00);			// Set UCSZ02:1 as 8-bit character data

}

unsigned char USART_receive(void) {	// Function to receive ASCII value from UDR0

while (!(UCSR0A & (1 << RXC0)));	// Keep Checking until RXC0 is 'High' to break

return UDR0;				// Return received serial into unsigned char

}

void USART_send(unsigned char data) {	// Function to transmit ASCII value into UDR0

while (!(UCSR0A & (1 << UDRE0)));	// Check until UDRE0 data register 'High'

UDR0 = data;				// Store unsigned char serial data into UDR0

}

void USART_putstring(char* StringPtr) {	// Function to break string into chars, and send

while (*StringPtr != 0x00) {	// Keep Looping until String Completed (null)

USART_send(*StringPtr);	// Send char value pointed by the string pointer

StringPtr++;			// Increment pointer to next char array location

}

}

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Figure 2 – Output Terminal of Serial Data

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Figure 3 – Xplained Mini only for Serial Communication

- VIDEO LINKS OF EACH DEMO

N/A (Not required for assignment)

- GITHUB LINK OF THIS DA

https://github.com/katmando

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Alex Cater